# Kinematics of the myCobot 280 — A Worked Book

Forward & inverse kinematics, the velocity-propagation Jacobian, manipulability, and the code — in reference notation

For: Abhishek Roy

2026

# Preface — how to read this book

This book explains the mathematics and the code of the myCobot 280 projects, in the notation of your reference notes (modified Denavit–Hartenberg, the velocity-propagation method). Read a chapter, then open the file it names and match each equation to the lines of code.

Two kinds of callouts appear throughout:

Key point. The one idea you must not miss.

Meaning. A plain-language reading of the symbols right after each equation.

Notation used throughout (from your reference notes):

A leading super/subscript denotes the frame: the first symbol above is the transform from frame i​−​1 to frame i; the second is an angular velocity of link i, written in frame i. A dot is a time derivative (joint speed), a hat is a unit vector (the z-axis).

# Chapter 1 — The robot and its frames

The myCobot 280 is a 6-joint (6-DOF) arm; every joint is revolute and turns about its own z-axis. To do kinematics we attach a coordinate frame to each link and describe how consecutive frames relate. We use the modified (Craig) Denavit–Hartenberg convention, which needs only two axes per frame (x and z) and four numbers per joint:

| symbol | name | meaning |
| --- | --- | --- |
| αi−1 | link twist | angle from Zi−1 to Zi about Xi−1 |
| ai−1 | link length | distance between the two z-axes along Xi−1 |
| di | link offset | distance from Xi−1 to Xi along Zi |
| θi | joint angle | angle from Xi−1 to Xi about Zi (the variable for a revolute joint) |

Key point. For a revolute joint only θi changes as the motor turns; αi−1,ai−1,di are fixed by the robot’s geometry.

Meaning. Twist and length describe how one joint axis sits relative to the next; offset and angle describe how far and how much you travel along/around the joint axis to reach the next link.

# Chapter 2 — Forward kinematics

Question. Given the six joint angles, where is the tool?

Each pair of frames is related by one modified-DH link transform — exactly the matrix in your notes:

with cθ=cosθ, sθ=sinθ. Chaining the six link transforms from the base to the tool gives the full forward kinematics:

Key point. Forward kinematics is just a product of these link matrices. One set of joint angles gives exactly one tool pose.

Meaning. The top-left 3×3 block of the tool transform T is the tool’s orientation; the top-right 3×1 block is its position.

In code (mycobot_thesis/robot.py): _dh_link(alpha, a, d, theta) builds one matrix above; fk_frames(q) multiplies them and returns every frame; and forward_kinematics(q) returns the last one, ​N0T. The DH table (identified from the URDF, verified to 10−15 m) is the array DH_TABLE.

# Chapter 3 — The Jacobian (velocity-propagation method)

Question. How does the tool move when the joints move? The Jacobian Jq answers this, mapping joint speeds q to the tool’s linear and angular velocity (its twist).

Your notes build J by propagating velocity outward, frame by frame. Angular velocity carries out and gains the new joint’s spin:

Linear velocity carries out, picking up the lever-arm cross-product, and gains a prismatic slide (zero for our revolute joints):

Meaning. The rotation R carries a vector from frame i into frame i+1; the cross-product ω×P is the velocity a rotating link imparts at its far end; θi+1 and di+1 add the new joint’s own motion.

For an all-revolute arm this closes into the compact geometric Jacobian, whose i-th column is

Key point. Column i says: if only joint i spins, the tool’s linear velocity is Zi×Pe−Pi and its angular velocity is Zi.

In code (robot.py): jacobian(q) builds exactly these columns from the frame axes zi and origins pi returned by fk_frames. It is verified against a finite-difference Jacobian to ≈2.5×10−7.

# Chapter 4 — Inverse kinematics

Question. We want the tool at a desired pose — which joint angles get there? There is no simple formula, so we iterate with damped least squares:

where e is the 6-vector pose error (position + orientation) and λ is a small damping constant.

Key point. The damping λ2I is what keeps the step finite near a singularity (where J loses rank and a plain inverse blows up). It trades a tiny steady-state error for stability.

Meaning. Each step multiplies the pose error by a damped inverse of the Jacobian to get a joint correction, then repeats until the tool reaches the target.

In code (mycobot_thesis/ik.py): damped_least_squares(...) is the equation above; pseudoinverse, jacobian_transpose, and levenberg_marquardt are the alternatives compared in the thesis. Damped least squares was the most robust (98.7% success); see the figure below.

# Chapter 5 — Manipulability and the task-constrained workspace

Not every reachable point is equally good, and — for drawing — not every reachable point can be reached with the pen perpendicular to the surface.

Dexterity. Yoshikawa manipulability and the condition number measure how far a configuration is from a singularity:

Meaning. w→0 (and κ→∞) at a singularity: the arm loses a direction of motion and IK becomes twitchy. Large w, small κ = dexterous.

Drawable workspace. Given a surface with normal n, the positions the pen can reach while staying perpendicular form the task-constrained set

Key point. The reachable set R and the drawable set Dn are different: Dn⊂R, and the gap depends on the surface orientation.

For the myCobot: only 57.9% of R allows a perpendicular pen on a vertical surface (35.2% front-facing), and 63.5% allows pen-down drawing on a horizontal table — an annulus around a dead zone above the base.

# Chapter 6 — Reading the code

Every module mirrors a chapter. The library is pure math (no ROS); the nodes wrap it for live RViz.

mycobot_thesis/robot.py — the model (Chapters 2–3, 5).

- DH_TABLE — the identified modified-DH parameters αi−1,ai−1,di,θioff.

- _dh_link, fk_frames, forward_kinematics — the link matrix and the product.

- jacobian — the velocity-propagation columns.

- manipulability, condition_number — the dexterity metrics.

- target_orientation(normal, advance) — builds the pen-perpendicular target frame.

mycobot_thesis/ik.py — the solvers (Chapter 4). damped_least_squares and the three alternatives, each returning convergence info.

mycobot_thesis/draw_node.py — the ROS driver. It picks a surface preset, generates the sine targets, solves IK for each (warm-started), and streams /joint_states. The 'paper' preset uses a raised IK branch so the arm is opened toward z while the pen traces the identical sine.

Key point. A ROS node is a thin wrapper: it calls the same robot/ik library the offline analysis uses, then publish()es. Same math, live robot.

Redundancy in action. A 6-DOF arm can reach the same tool pose with different postures (IK branches). Seeding the IK toward a raised branch keeps the drawn sine identical while lifting the elbow:

The live demo (ros2 launch mycobot_thesis thesis_draw.launch.py surface:=paper) writes the sine left-to-right on a table in front of the robot, raised posture:

# Appendix — Identified DH table and commands

| i | αi−1 (°) | ai−1 (m) | di (m) | θioff (°) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0.13056 | +90 |
| 2 | +90 | 0 | 0 | −90 |
| 3 | 0 | −0.1104 | 0 | 0 |
| 4 | 0 | −0.096 | 0.06062 | −90 |
| 5 | +90 | 0 | 0.07318 | +90 |
| 6 | −90 | 0 | 0.0456 | 0 |

# analysis (pure Python)
cd ~/ros2_ws/src/mycobot_thesis
python3 analysis/exp1_feasibility.py        # task-constrained workspace
python3 analysis/exp2_ik_comparison.py      # IK solver comparison
# live drawing in RViz
cd ~/ros2_ws && colcon build --packages-select mycobot_thesis && source install/setup.bash
bash src/mycobot_thesis/scripts/run_thesis_demo.sh surface:=paper