# Task-Constrained Workspace and Inverse-Kinematics Analysis for Surface-Tracing with a 6-DOF Manipulator (myCobot 280)

Abhishek Roy

2026

# Abstract

Drawing or writing on a surface requires a manipulator not only to reach a sequence of Cartesian points but to reach them while holding the tool normal to the surface. This orientation constraint shrinks the usable workspace in a way that depends on the surface geometry. We formalise the task-constrained (drawable) workspace for surface-tracing and analyse it for the myCobot 280, a small 6-DOF arm, using a modified Denavit–Hartenberg model whose forward kinematics matches the manufacturer URDF to 10−15 m. We map, over the reachable workspace, where the pen can be held perpendicular to vertical and horizontal surfaces and in which direction; we compare four Jacobian-based inverse-kinematics solvers on the full-pose tracing task; and we relate feasibility to manipulability and conditioning. We find that only 57.9% of the reachable workspace admits a perpendicular pen on a vertical surface (and only 35.2% in the outward direction), while 63.5% admits pen-down drawing on a horizontal surface, in an annular region that excludes a low-manipulability zone directly above the base. We demonstrate sub-millimetre sine tracing on both a front-facing vertical board and a horizontal table at analysis-selected locations.

# I. Introduction

A recurring robotics task is to trace a planar curve — a signature, a calibration pattern, a sine wave — on a physical surface with a pen or tool. Unlike free-space motion, the tool must remain perpendicular to the surface at every point, adding an orientation constraint to the usual position requirement. For a small manipulator this constraint is not a formality: many positions the end-effector can reach it cannot reach with the required orientation.

This paper studies that gap for the myCobot 280, a 6-DOF revolute arm with a reach of roughly 0.30 m. Our contributions are:

- a verified modified-DH kinematic model and a velocity-propagation Jacobian (Section II);

- a mapping of the task-constrained workspace — where and in which direction the pen can be held normal to vertical and horizontal surfaces (Section III);

- a quantitative comparison of four inverse-kinematics solvers on the constrained tracing task (Section IV);

- manipulability and conditioning maps that explain why the drawable region has the shape it does (Section V); and

- sub-millimetre demonstrations on a front-facing vertical board and a horizontal table (Section VI).

# II. Kinematic Model

## A. Forward kinematics

We use the modified (Craig) Denavit–Hartenberg convention. The transform from frame i−1 to frame i is

Ai=Rotxαi−1 Transxai−1 Rotzθi Transzdi,

which in homogeneous form is

Ai=cθi−sθi0ai−1sθicαi−1cθicαi−1−sαi−1−sαi−1disθisαi−1cθisαi−1cαi−1cαi−1di0001,

with c⋅=cos, s⋅=sin. The base-to-tool transform is the product Tq=A1A2A3A4A5A6.

Because the URDF link frames are not placed on DH axes, the DH parameters were identified by fitting αi−1,ai−1,di,θioff so that Tq reproduces the URDF forward kinematics; the residual is max∥TDH−TURDF∥≈10−15 m over 5000 random configurations. Table I lists the result.

Table I. Identified modified-DH parameters of the myCobot 280.

| i | αi−1 (°) | ai−1 (m) | di (m) | θioff (°) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0.13056 | +90 |
| 2 | +90 | 0 | 0 | −90 |
| 3 | 0 | −0.1104 | 0 | 0 |
| 4 | 0 | −0.096 | 0.06062 | −90 |
| 5 | +90 | 0 | 0.07318 | +90 |
| 6 | −90 | 0 | 0.0456 | 0 |

## B. Jacobian by velocity propagation

Propagating link velocities outward yields, for the all-revolute arm, the geometric Jacobian whose i-th column is

Jiq=zi×pe−pizi,

where zi and pi are the axis and origin of joint i in the base frame and pe is the tool position. It maps joint rates to the tool twist, vω=Jq q. Equation (3) matches a finite-difference Jacobian to ≈2.5×10−7.

## C. Dexterity metrics

We quantify local dexterity by the Yoshikawa manipulability and the condition number of the translational Jacobian Jv (top three rows of J):

wq=det​JvJv⊤,  κq=σmaxJvσminJv.

w→0 and κ→∞ mark singular, poorly-conditioned configurations.

# III. Task-Constrained Workspace

## A. Definition and method

Let R be the reachable workspace (positions attainable with some joint vector within limits). Given a surface with unit normal n, the drawable (task-constrained) workspace is

Dn={ p∈R:∃ q, fkposq=p, zq=±n },

i.e. positions the tool can reach while its approach axis z is aligned with the surface normal. We estimate Dn by sampling a grid of positions and testing, for each candidate normal direction and from several seed configurations, whether damped-least-squares IK converges with position error below 2 mm and orientation error below 3×10−2.

## B. Results

Task-constrained workspace. (a) On a vertical surface, only the back (toward the robot) pen direction is feasible near the base, while the front (away) direction opens up only in the outer region; a narrow band admits both. (b–c) On a horizontal table the drawable region is an annulus that excludes a zone directly above the base.

Figure 1 maps D. On a vertical surface, only 57.9% of R admits a perpendicular pen, and only 35.2% admits the front (outward) direction. This explains a practical observation: a board placed close to the robot can only be drawn on with the pen facing back; to draw with the pen facing the board from the front, the board must be placed in the outer region (x≳0.17 m). On a horizontal table, 63.5% of the reachable cells admit pen-down drawing, but the feasible set is an annulus: the region directly above the base is reachable yet not drawable, because the wrist cannot point straight down there.

# IV. Inverse-Kinematics Comparison

## A. Solvers

All solvers iterate q←q+Δq with the stacked pose error e= pt−pq; eori , clamped to joint limits. The updates are:

ΔqDLS=J⊤​JJ⊤+λ2I−1e,  Δqpinv=J†e,

ΔqJT=α J⊤e, α=⟨e,JJ⊤e⟩⟨JJ⊤e,JJ⊤e⟩,

with Levenberg–Marquardt using (6) with λ adapted to the error trend. Damped least squares (6) trades a small steady-state error for stability near singularities; the pseudoinverse converges quadratically but is ill-behaved where J loses rank.

## B. Results

On 150 reachable target poses seeded from a common ready configuration:

Table II. IK solver comparison on the full-pose tracing task.

| Solver | Success | Mean iters | Mean time (ms) |
| --- | --- | --- | --- |
| Damped least squares | 98.7 % | 76 | 15 |
| Pseudoinverse | 26 % | 8 | 30 |
| Jacobian-transpose | 29 % | 2000 | 365 |
| Levenberg–Marquardt | 61 % | 10 | 16 |

Inverse-kinematics comparison. (a) success rate, (b) iterations, (c) solve time, (d) error-vs-iteration for one pose. Damped least squares is the most robust; pseudoinverse and Levenberg–Marquardt converge quadratically but are brittle near singularities; Jacobian-transpose always progresses but is an order of magnitude slower.

Figure 2 shows the trade-off: damped least squares is by far the most robust (98.7%), at the cost of more iterations and a damping-limited accuracy; the pseudoinverse and Levenberg–Marquardt reach machine precision in ∼10 iterations on well-conditioned poses but fail more often near singularities; the Jacobian-transpose is uniformly slow. For surface tracing — a sequence of small, well-seeded steps — DLS is the appropriate default.

# V. Manipulability and Conditioning

Manipulability w and condition number κ. A low-manipulability zone sits directly above the base; a dexterous ring lies at mid-radius; conditioning degrades toward the workspace boundary.

Figure 3 explains the shape of the drawable set. Manipulability w is near zero directly above the base (the arm is close to a shoulder/wrist singularity there) and peaks in a mid-radius ring; the condition number κ grows toward the workspace boundary, where the arm approaches full extension. The drawable annulus of Figure 1(b–c) coincides with this high-w, low-κ ring — feasibility and good conditioning are geometrically the same region.

# VI. Demonstrations

Sine tracing placed by the analysis. (a) A front-facing vertical board (pen away from the robot) in the outer feasible region; (b) a horizontal table (pen down) in the dexterous ring. Both track the target to 0.000 mm.

Using the maps of Sections III and V to choose surface placements, the arm traces a two-cycle sine to a maximum position error of 0.000 mm on both a front-facing vertical board at 0.21,0,0.12 m and a horizontal table at 0,0.18,0.10 m (Figure 4), confirming that the constrained-workspace maps predict where clean drawing is possible.

# VII. Conclusion

The reachable and drawable workspaces of a surface-tracing manipulator are distinct sets whose difference depends on the surface normal. For the myCobot 280, a perpendicular pen is feasible over only 57.9% (vertical) and 63.5% (horizontal) of the reachable workspace, and the feasible regions coincide with the arm’s high-manipulability ring. Damped least squares is the most robust IK method for the task. Future work: extending the analysis to arbitrary tilted surfaces and full tool paths, exploiting redundancy for manipulability-optimal drawing, and validating on the physical robot and in Gazebo.

# References

- J. J. Craig, Introduction to Robotics: Mechanics and Control, 4th ed. Pearson, 2018.

- T. Yoshikawa, “Manipulability of robotic mechanisms,” Int. J. Robotics Research, 4(2), 1985.

- C. W. Wampler, “Manipulator inverse kinematic solutions based on vector formulations and damped least-squares methods,” IEEE Trans. SMC, 16(1), 1986.

- Elephant Robotics, myCobot 280 URDF / description, 2023.